This Planning Guide has been prepared by the City of Greater Geelong to provide applicants with some key principles for undertaking works near trees located within road reserves and public spaces. In particular, it outlines minimum distances that need to be maintained between proposed vehicle crossings or buildings and these trees. Council aims to preserve and protect existing trees located in road reserves and public spaces. They play an important role in the amenity of our streets and parks, as well as contributing to local biodiversity. Some new developments have the potential to damage or require the removal of these trees, particularly when new crossovers are proposed.

At the outset of designing a new development, the location of any existing street or park trees near the site need to be considered. They also need to be shown on the plans lodged as part of the planning permit application. It is important to remember that the removal of Council’s tree assets will not automatically be approved. Many factors inform Council’s decision on whether a street or park tree can be removed. The removal of each tree is assessed on its own merits by Council’s Tree Management Unit.

The Australian Standard AS4970 – 2009 Protection of Trees on Development Sites is a key document to guide Council’s decisions. It requires protection zones be used to safeguard trees during the construction phase. Figure 1 on the reverse side of this Planning Guide show indicative protection zones. The table below is as an indicative guide only, however it may assist in the early design phases of the development:

| Trunk Diameter - mm (measured 1.4m above ground) | Tree Protection Zone (TPZ) | Structural Root Zone (SRZ) (Approx) |
| --- | --- | --- |
| 200 | 2.4m | 1.7m |
| 400 | 4.8m | 2.2m |
| 600 | 7.2m | 2.7m |
| 800 | 9.6m | 3.0m |
| 1000 | 12m | 3.4m |

The Structural Root Zone (SRZ) is a ‘No Go Zone’ and works that encroach into the SRZ will not be allowed.  Sometimes Council does allow works to encroach into the Tree Protection Zone (TPZ), although this is dependant upon site specific factors.

Where trees are proposed to be retained, Council must be satisfied that the works proposed will not affect the long term health of the tree. In situations where Council does agree to the removal of a tree, the applicant will be responsible for meeting all costs associated with the removal of the tree, offset replanting and the care of the newly planted trees for two years.  The number of replacement trees is based on the size, age, and type of tree being removed.  All works are undertaken by Council or its approved contractors on the applicant’s behalf.

If you’re not sure whether your proposal will affect a street or park tree, or want to know if Council will be likely to approve the removal of a particular tree, Contact Council’s Statutory Planning Unit on (03) 5272 4456.

Figure 1: Indicative Tree Protection Zones