|  | # gis formats at city of greater geelong The purpose of these guidelines is to provide information about the gis data formats that can be returned to city of greater geelong (COGG) |  |
| --- | --- | --- |
|  | # GIS at city of greater geelong The City of Greater Geelong has a highly functional and detailed GIS system that is used widely across council. This document aims to assist council contractors and consultants of the formats that can be provided back to council which may then integrate back into the councils GIS system. # gis platform details City of Greater Geelong uses Environment System Research Institute (ESRI) platform. The ESRI platform is one of the most widely used and accepted systems in the world. The formats listed below should be projected using the map projection listed in this document. ### Acceptable Vector Formats - ESRI Shapefile - .shp - File Geodatabase - .gdb - Personal Geodatabase - .mdb ### Acceptable Raster Formats - GeoTiff - .tif - Jpeg/Jpeg2000 – .jpg - ECW - .ecw Raster data should be in the most appropriate image resolution according to each project’s desired outcome. Each raster image should also feature an associated world file (.tfw, .jgw) to ensure the geo-reference of the images is correct. | # Map Projection Details All of the City of Greater Geelong's spatial geodatabase PLACES is required to be projected using the Map Grid of Australia (MGA) Zone 55 on the Geocentric Datum of Australia 1994 (GDA94). MGA Zone 55 ---------------- Projected Coordinate System: Name: GDA 1994 MGA Zone 55 Projection: Transverse Mercator Parameters: False Easting: 500000 False Northing: 10000000 Central Meridian: 147.000000 Scale Factor: 0.999600 Latitude Of Origin: 0.000000 Linear Unit: Meter (1.000000) GDA 94 --------- Geographic Coordinate System: Name: GCS GDA 1994 Angular Unit: Degree (0.017453292519943299) Prime Meridian: Greenwich (0.000000000000000000) Datum: D GDA 1994 Spheroid: GRS 1980 Semimajor Axis: 6378137.000000000000000000 Semiminor Axis: 6356752.314140356100000000 Inverse Flattening: 298.257222101000020000 # contact Digital files smaller than 20MB may be delivered by email to gis@geelongcity.vic.gov.au For files larger than this please use a web-based file sharing service such as DropBox or YouSendIt. Files on CDs or USB device can be sent to: GIS Unit Information Services Department, SIS Unit 17 Gheringhap St Geelong VIC 3220 |