CLAUSE 54 ASSESSMENT – One Dwelling or Second Small Dwelling on a Lot less than 300sqm in Area

APPLICATION REQUIREMENTS

An application to which this clause applies must be accompanied by:

- A site description – Clause 54.01-1.

- A design response – Clause 54.01-2 including a landscape plan.

- A written statement outlining which standards are met and which are not met. If a standard is not met, the written statement must include an explanation of how the development meets the corresponding objective having regard to the corresponding decision guidelines.

CLAUSE 54 ASSESSMENT

The following objectives and standards are current up to and including Amendment VC282 on 13/8/2025.

54.02 Neighbourhood character

| 54.02-1 Street setback objective To ensure that the setbacks of buildings from a street respond to the existing or preferred neighbourhood character and make efficient use of the site. | Standard A2-1 Walls of buildings are set back from streets: - At least the distance specified in a schedule to the zone if the distance specified in the schedule is less than the distance specified in Table A2-1; or - If no distance is specified in a schedule to the zone, the distance specified in Table A2-1. Porches, pergolas and verandahs that are less than 3.6 metres high and eaves may encroach not more than 2.5 metres into the setbacks of this standard. | Deemed to Comply The lessor front wall setback of the existing buildings on the abutting allotments is INSERT metres, which results in a requirement for an INSERT metre street setback. This has been achieved as the building is setback INSERT metres from the street. OR The lessor setback of the adjoining building is INSERT metres which is greater than the lessor 6 metre setback therefore the setback must be 6m. This has been achieved as the building is setback INSERT metres from the street. OR One adjoining lot is vacant and the other lot has a building setback INSERT metres from the street. The proposed setback complies as it is the same setback of the adjoining dwelling at INSERT metres/is setback 6m which is the lessor setback. OR Both adjoining lots are vacant. The setback of INSERT metres meets the Transport Zone 2/other street setback. OR The site is located on a corner lot and the setback of INSERT metres meets the setback of the adjoining building/is the lessor setback. The side setback is INSERT metres which meets the setback of the adjoining building face the side street/is the lessor setback. The front wall of the development fronting the side street is setback INSERT metres which meets the setback of the adjoining building facing the side street/is the lessor setback. Not Applicable The application proposes construction of new small second dwelling behind an existing dwelling such that the established front setback is to be retained. Variation INSERT Before deciding on an application, the responsible authority must consider: - Any relevant neighbourhood character objective, policy or statement set out in this scheme. - The design response. - Whether the siting of the building is constrained by the shape, dimensions, slope or other conditions of the site. - Whether a different setback would be more appropriate taking into account the prevailing setbacks of existing buildings on nearby lots. - The visual impact of the building when viewed from the street and from adjoining properties. - Whether a different setback affects the ability to retain or plant canopy trees. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met Yes, standard varied |
| --- | --- | --- | --- |
| 54.02-2 Building height objective To ensure that the height of buildings responds the existing or preferred neighbourhood character. | Standard A2-2 The maximum building height does not exceed the maximum height specified in the zone, schedule to the zone or an overlay that applies to the land. If no maximum height is specified in the zone, schedule to the zone or an overlay, the maximum building height does not exceed 9 metres, unless the slope of the natural ground level at any cross section wider than 8 metres of the site of the building is 2.5 degrees or more, in which case the maximum building height should not exceed 10 metres. | Deemed to Comply The maximum building height applicable to the site is INSERT metres and INSERT storeys at any point (excluding a basement). The proposed maximum building height of INSERT metres meets the standard/mandatory building height requirement of the ZONE. Variation The maximum building height applicable to the site is INSERT metres which exceeds the Standard by INSERT metres Before deciding on an application, the responsible authority must consider: - Any relevant neighbourhood character objective, policy or statement set out in this scheme. - Any maximum building height specified in the zone, a schedule to the zone or an overlay applying to the land. - The design response. - The effect of the slope of the site on the height of the building. - The relationship between the proposed building height and the height of existing adjacent buildings. - The visual impact of the building when viewed from the street and from adjoining properties. INSERT The dwellings are single storey in scale in line with surrounding development. While the dwellings are double storey in scale, the design solution includes single storey elements in response to the scale of the surrounding area. Does not meet the objective INSERT | Right of Appeal No, standard met Yes, standard varied |
| 54.02-3 Side and rear setbacks objective To ensure that the height and setback of a building from a boundary responds to the existing or preferred neighbourhood character and limits the impact on the amenity of existing dwellings or small second dwellings. | Standard A2-3 A new building not on or within 200mm of a boundary is set back from side or rear boundaries at least 1 metre, plus 0.3 metres for every metre of height over 3.6 metres up to 6.9 metres, plus 1 metre for every metre of height over 6.9 metres. Sunblinds, verandahs, porches, eaves, facias, gutters, masonry chimneys, flues, pipes, domestic fuel or water tanks, and heating or cooling equipment or other services may encroach not more than 0.5 metres into the side and rear setbacks. Landings that have an area of not more than 2 square metres and less than 1 metre high, stairways, ramps, pergolas, shade sails and carports may encroach into the side and rear setbacks. | Deemed to Comply The required setbacks have been achieved for the development and all side and rear setbacks comply with the A2-3 formula contained in the Standard. Variation INSERT Before deciding on an application, the responsible authority must consider: - Any relevant neighbourhood character objective, policy or statement set out in this scheme. - The design response. - The impact on the amenity of the habitable room windows and secluded private open space of existing dwellings or small second dwellings. - Whether the wall is opposite an existing or simultaneously constructed wall built to the boundary. - Whether the wall abuts a side or rear lane. - Whether a different setback in a rear yard affects the ability to retain or plant canopy trees. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met Yes, standard varied |
| 54.02-4 Walls on boundaries objective To ensure that the location, length and height of a wall on a boundary responds to the existing or preferred neighbourhood character and limits the impact on the amenity of existing dwellings and small second dwellings. | Standard A2-4 A new wall constructed on or within 200mm of a side or rear boundary of a lot or a carport constructed on or within 1 metre of a side or rear boundary of lot does not abut the boundary for a length that exceeds the greater of the following distances: - 10 metres plus 25 per cent of the remaining length of the boundary of an adjoining lot, or - The length of existing or simultaneously constructed walls or carports abutting the boundary on an abutting lot. A new wall or carport may fully abut a side or rear boundary where slope and retaining walls or fences would result in the effective height of the wall or carport being less than 2 metres on the abutting property boundary. A building on a boundary includes a building set back up to 200mm from a boundary. The height of a new wall constructed on or within 200mm of a side or rear boundary or a carport constructed on or within 1 metre of a side or rear boundary does not exceed an average of 3.2 metres with no part higher than 3.6 metres unless abutting a higher existing or simultaneously constructed wall. | Deemed to Comply The proposed wall on the INSERT boundary extends for a length of INSERT metres, has an average height of INSERT metres, and a maximum height of INSERT metres. There are no carports proposed to be on or within 1 metre of a side or rear boundary. Not Applicable No new walls on or within 200mm of a boundary, or carport within 1m of a boundary are proposed as part of this application. Variation The proposed wall on or within 200mm of the INSERT boundary extends for a length of INSERT metres, has an average height of INSERT metres, and a maximum height of INSERT metres. This wall exceeds the requirements of the Standard. The proposed carport within 1 metre of the INSERT boundary extends for a length of INSERT metres, has an average height of INSERT metres, and a maximum height of INSERT metres. This carport exceeds the requirements of the Standard. Before deciding on an application, the responsible authority must consider: - Any relevant neighbourhood character objective, policy or statement set out in this scheme. - The design response. - The extent to which walls on boundaries are part of the neighbourhood character. - The impact on the amenity of existing dwellings or small second dwellings. - The opportunity to minimise the length of walls on boundaries by aligning a new wall on a boundary with an existing wall on a lot of an adjoining property. - The orientation of the boundary that the wall is being built on. - The width of the lot. - The extent to which the slope and retaining walls or fences reduce the effective height of the wall. - Whether the wall abuts a side or rear lane. - The need to increase the wall height to screen a box gutter. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met Yes, standard varied |
| 54.02-5 Site coverage objective To ensure that the site coverage responds to the existing or preferred neighbourhood character and responds to the features of the site. | Standard A2-5 The site area covered by buildings does not exceed: - The maximum site coverage specified in a schedule to a zone; or - If no maximum site coverage specified in a schedule to a zone, the percentage specified in Table A2-5: | Deemed to Comply The maximum site coverage specified for this site is INSERT percent as per the schedule to the zone/Table A2-5. The proposed site coverage of buildings is INSERT percent of the site, which meets the standard. Variation The maximum site coverage specified for this site is INSERT percent. The proposed site coverage of buildings is INSERT percent of the site, which is greater than the maximum required by the standard. Before deciding on an application, the responsible authority must consider: - Any relevant neighbourhood character objective, policy or statement set out in this scheme. - The design response. - The existing site coverage and any constraints imposed by existing development or the features of the site. - The site coverage of adjacent properties. - The effect of the visual bulk of the building and whether this is acceptable in the neighbourhood. - Whether a different area of site coverage affects the ability to retain or plant canopy trees. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met Yes, standard varied |
| 54.02-6 Tree canopy objective To provide tree canopy that responds to the neighbourhood character of the area and reduces the visual impact of buildings on the streetscape. To preserve existing canopy cover and support the provision of new canopy cover. To ensure new canopy trees are climate responsive, support biodiversity, wellbeing and amenity, and help reduce urban heat | Standard A2-6 Provide a minimum canopy cover as specified in Table A2-6. A tree must meet the following: - Reach a height of at least 6 metres at maturity. - Achieve a canopy width of at least 4 metres at maturity. - Planted in a minimum deep soil area of 12 square metres with a minimum plan dimension 2.5 metres or in a planter with a minimum volume of 12 cubic metres with a minimum depth of 0.8 metres of planter soil Existing trees to be retained meet all of the following: - Has a height of at least 5 metres, - Has a trunk circumference of 0.5 metres or greater at 1.4 metres above ground level, - Has a trunk that is located at least 4 metres from proposed buildings. Existing trees that are retained can be used to satisfy the tree canopy requirement. Any tree required to be planted under this standard must be of species to the satisfaction of the responsible authority, having regard to the location and relevant geographic factors. | Deemed to Comply There are adequate areas available on site to allow for tree retention/planting which will complement the landscape character of the area. The tree canopy requirement has been achieved as per Table A2-6 though the planting of 1 tree/2 trees/3 trees. An appropriate landscaping plan has been submitted for assessment. Variation INSERT Before deciding on an application, the responsible authority must consider: - Any relevant neighbourhood character objective, policy or statement set out in this scheme. - The site context and design response. - The extent to which the existing and proposed canopy trees contribute to a greener environment and reduce urban heat. - Whether the growth characteristics of existing trees and proposed canopy trees will provide the required canopy cover. - The suitability of the planting location, deep soil areas and planter soil volume for proposed canopy trees. - Whether the species of canopy tree is suited to the soil conditions of the site. - Whether an alternative combination of the canopy trees will provide the required tree canopy cover. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met Yes, standard varied |
| 54.02-7 Front Fences To encourage front fence design that responds the existing or preferred neighbourhood character. . | Standard A2-7 A front fence within 3 metres of a street is: - The maximum height specified in a schedule to the zone, or - If no maximum height is specified in a schedule to the zone, the maximum height specified in Table A2-7. | N/A This Standard does not apply to a small second dwelling. Deemed to Comply The front fence is 1.5m (other Street)/2m (Streets in a Transport Zone 2) in height. Not Applicable No front fence is proposed as part of this permit application. Variation INSERT Before deciding on an application, the responsible authority must consider: - Any relevant neighbourhood character objective, policy or statement set out in this scheme. - The design response. - The setback, height and appearance of front fences on adjacent properties. - The extent to which slope and retaining walls reduce the effective height of the front fence. - Whether the fence is needed to minimise noise intrusion. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met No, standard not relevant Yes, standard varied |
| 54.02-8 Building setback for small second dwelling To ensure that small second dwellings are sited to respond to the existing or preferred neighbourhood character. | Standard A2-8 Walls of a small second dwelling are set back behind the front wall of the existing dwelling on the lot, facing the frontage. Porches, pergolas, verandahs, and eaves do not encroach into the setback of this standard | N/A This Standard does not apply to a dwelling. Deemed to Comply Walls of the small second dwelling are set back behind the front wall of the existing dwelling on the lot, facing the frontage and porches, pergolas, verandahs, and eaves do not encroach into the setback. Variation INSERT Before deciding on an application, the responsible authority must consider: - Any relevant neighbourhood character objective, policy or statement set out in this scheme. - The design response. - The setback, height and appearance of front fences on adjacent properties. - The extent to which slope and retaining walls reduce the effective height of the front fence. - Whether the fence is needed to minimise noise intrusion. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met No, standard not relevant Yes, standard varied |

54.03 Liveability

| 54.03-1 Street integration objective To integrate the layout of development with the street to support the safety and amenity of residents | Standard A3-1 Where a development fronts a street, a vehicle accessway or abuts public open space passive surveillance is provided by a direct view from a balcony or a habitable room window to each street, vehicle accessway and public open space. This Standard does not apply to a small second dwelling. | N/A This Standard does not apply to a small second dwelling. Deemed to Comply The development is designed to provide passive surveillance of INSERT Street/a vehicle accessway/public open space through a direct view from a balcony or habitable room window from the dwellings INSERT. Variation INSERT Before deciding on an application, the responsible authority must consider: - Any relevant neighbourhood character objective, policy or statement set out in this scheme. - The design response. INSERT Does not meet the objective INSERT | Right of Appeal No |
| --- | --- | --- | --- |
| 54.03-2 Private open space objective To provide adequate private open space for the reasonable recreation and service needs of residents. | Standard A3-2 A dwelling or small second dwelling has private open space of an area and dimensions specified in the schedule to the zone. If no area or dimensions are specified in the schedule to the zone, a dwelling or residential building has private open space with direct access from a living room, dining area or kitchen consisting of: - An area of 20 per cent of the area of the lot, but not less than 25 square metres. At least one part of the private open space consists of secluded private open space with a minimum area of 25 square metres and a minimum dimension of 3 metres width; or - A balcony with at least the area and dimensions specified in Table A3-2; or - An area on a roof of at least 10 square metres with a minimum width of 2 metres. If the area and dimensions of the private open space or secluded private open space is specified in a schedule to the zone: - The area and dimensions specified for the private open space and secluded private open space must be less than the area and dimensions specified in this standard, and - The area and dimensions specified for a balcony or an area on a roof must be less than the area and dimensions specified in this standard. A small second dwelling has a secluded private open space consisting of an area of 8 square metres with a minimum dimension of 1.6 metres and convenient access from a living area, dining area or kitchen If a cooling or heating unit is located in the secluded private open space or private open space the required area is increased by 1.5 square metres. Where ground level private open space is provided, an area for clothes drying is provided. | Deemed to Comply Schedule INSERT to the ZONE requires INSERT sqm/There is no area specified in a schedule to the zone. An area of 25 square metres of secluded private open space with a minimum dimension of 3 metres with direct access from a living room/dining area/kitchen has been provided. A balcony area of at least INSERT square metres with a minimum dimension of INSERT metres with direct access from a living room/dining area/kitchen has been provided in accordance with Table A3-2. A roof area of at least 10 square metres and a minimum width of 2 metres with direct access from a living room/dining area/kitchen has been provided. The small second dwelling has been provided with 8sqm with a minimum width of 1.6m with convenient access from a living room/dining area/kitchen An additional 1.5 square metres has been provided as a cooling/heating unit is located in the secluded private open space or private open space areas of the dwelling/small second dwelling. Clothes drying has been provided in the ground level private open space of the dwelling/small second dwelling. Variation INSERT Before deciding on an application, the responsible authority must consider: - The design response. - The useability of the private open space, including its size and accessibility. - The availability of and access to public or communal open space. - The orientation of the lot to the street and the sun. INSERT Does not meet the objective INSERT | Right of Appeal No |
| 54.03-3 Solar access to open space objective To allow solar access into the secluded private open space of new dwellings. | Standard A3-3 The southern boundary of secluded private open space is set back from any wall on the north of the space at least (2+0.9h) metres, where ‘h’ is the height of the wall. | N/A This Standard does not apply to a small second dwelling. Deemed to Comply All proposed secluded private open space receives adequate solar access. The wall located on the northern side of the proposed secluded private open space is INSERT metres high, requiring the southern boundary of the proposed secluded private open space to be set back INSERT metres from the wall. The southern boundary is set back INSERT metres from the north wall in accordance with the standard. Variation INSERT Before deciding on an application, the responsible authority must consider: - The design response. - The useability and amenity of the secluded private open space based on the sunlight it will receive. INSERT Does not meet the objective INSERT | Right of Appeal No |
| 54.03-4 Daylight to new windows objective To allow adequate daylight into new habitable room windows. | Standard A3-4 A window in an external wall of the building is provided to all habitable rooms. Habitable rooms in a dwelling have a window that faces: - An outdoor space clear to the sky or a light court with a minimum area of 3 square metres and minimum dimension of 1 metre clear to the sky, not including land on an abutting lot; or - A verandah provided it is open for at least one third of its perimeter; or - A carport provided it has two or more open sides and is open for at least one third of its perimeter. | Deemed to Comply All habitable room windows are located on an external wall. All habitable room windows face: - An outdoor space clear to the sky or a light court with a minimum area of 3 square metres and minimum dimension of 1 metre clear to the sky, not including land on an abutting lot; or - A verandah provided it is open for at least one third of its perimeter; or - A carport provided it has two or more open sides and is open for at least one third of its perimeter. Variation INSERT Before deciding on an application, the responsible authority must consider: - The design response. - The extent to which habitable rooms are provided with reasonable daylight access through the number, size, location and orientation of windows. - The useability and amenity of the dwelling based on the layout, siting, size and orientation of habitable rooms. - Whether there are other windows in the habitable room which have access to daylight. INSERT Does not meet the objective INSERT | Right of Appeal No |
| 54.03-5 Safety and accessibility for small second dwelling objective To ensure access to a small second dwelling is safe, convenient and meets the needs of residents. | Standard A3-5 A small second dwelling is provided with a clear and unobstructed path from the frontage that: - Has a minimum width of at least 1 metre, with no encroachments. If the path is longer than 30 metres, the minimum width of the path is at least 1.8 metres. - Has a minimum clear height of at least 2 metres, with no encroachments. - Has a gradient no steeper than 1 in 14. - Has a cross fall no steeper than 1 in 40. - Is sealed or has an all-weather access. | N/A This Standard does not apply to a dwelling. Deemed to Comply The proposed path is clear and unobstructed, has a minimum width of 1m/1.8m with no encroachments, has a minimum clear height of 2m with no encroachments, has a gradient less than 1 in 14 or a cross fall no steeper than 1 in 40 and is sealed/has all weather access. Variation INSERT Before deciding on an application, the responsible authority must consider the safety and accessibility of the small second dwelling. INSERT Does not meet the objective INSERT | Right of Appeal No |

54.04 External Amenity

| 54.04-1 Daylight to existing windows objective To allow adequate daylight into existing habitable room windows. | Standard A4-1 Buildings opposite an existing habitable room window provide for a light court to the existing window that has a minimum area of 3 square metres and minimum dimension of 1 metre clear to the sky. The calculation of the area may include land on the abutting lot. Walls or carports more than 3 metres in height opposite an existing habitable room window are set back from the window at least 50 per cent of the height of the new wall if the wall is within a 55 degree arc from the centre of the existing window. The arc may be swung to within 35 degrees of the plane of the wall containing the existing window. Where the existing window is above ground floor level, the wall height is measured from the floor level of the room containing the window. | Deemed to Comply The required light court of 3 square metres with a minimum dimension of 1 metre clear to the sky has been achieved and all windows have access to the required level of daylight. The proposed wall/carport is more than 3m in height at INSERT metres. The wall/carport has been setback INSERT metres which is 50% of the proposed wall height. Not Applicable There are no existing habitable room windows opposite the proposed buildings that require consideration. Variation Before deciding on an application, the responsible authority must consider: - The design response. - The extent to which the existing dwelling or small second dwelling has provided for reasonable daylight access to its habitable rooms through the siting and orientation of its habitable room windows. - The impact on the amenity of existing dwellings. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met Yes, standard varied |
| --- | --- | --- | --- |
| 54.04-2 Existing north facing windows objective To allow adequate solar access to existing north-facing habitable room windows. | Standard A4-2 Where a north-facing habitable room window of a neighbouring dwelling or small second dwelling is within 3 metres of a boundary on an abutting lot a new building is to be set back from the boundary 1 metre, plus 0.6 metres for every metre of height over 3.6 metres up to 6.9 metres, plus 1 metre for every metre of height over 6.9 metres. This setback is to be provided for a distance of at least 3 metres from the edge of each side of the window. For this standard, a north-facing window is a window with an axis perpendicular to its surface oriented north 20 degrees west to north 30 degrees east. | Not Applicable There are no north facing habitable room windows on abutting lots that require consideration. Deemed to Comply There is a north facing window on the neighbouring dwelling to the INSERT. The proposed building has been setback INSERT meeting the standard. Variation INSERT Before deciding on an application, the responsible authority must consider: - The design response. - Existing sunlight to the north-facing habitable room window of the existing dwelling or small second dwelling. - The impact on the amenity of existing dwellings or small second dwellings. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met Yes, standard varied |
| 54.04-3 Overshadowing secluded open space objective To ensure buildings do not significantly overshadow existing secluded private open space. | Standard A4-3 The area of secluded private open space that is not overshadowed by the new development is greater than 50 per cent, or 25 square metres with a minimum dimension of 3 metres, whichever is the lesser area, for a minimum of five hours between 9 am and 3 pm on 22 September. If existing sunlight to the secluded private open space of an existing dwelling or small second dwelling is less than the requirements of this standard, the amount of sunlight will not be further reduced | Deemed to Comply The extent of overshadowing associated with the proposed development is within the requirements of the standard as: More than 50% of the SPOS or 25sqm with a minimum dimension of 3m is not overshadowed for a minimum of 5 hours between 9am and 3pm on 22 September Not Applicable There is no secluded private open space on adjoining land that requires consideration. Variation INSERT Before deciding on an application, the responsible authority must consider: - The design response. - The impact on the amenity of existing dwellings or small second dwelling. - Existing sunlight penetration to the secluded private open space of the existing dwelling or small second dwelling. - The time of day that sunlight will be available to the secluded private open space of the existing dwelling or small second dwelling. - The effect of a reduction in sunlight on the existing use of the existing secluded private open space. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met Yes, standard varied |
| 54.04-4 Overlooking objective To limit views into existing secluded private open space and habitable room windows. | Standard A4-4 In Clause 54.04-4 a habitable room does not include a bedroom. A habitable room window, balcony, podium, terrace, deck or patio is located and designed to avoid direct views into the secluded private open space of an existing dwelling or small second dwelling within a horizontal distance of 9 metres (measured at ground level) of the window, balcony, terrace, deck or patio. Views are measured within a 45 degree angle from the plane of the window or perimeter of the balcony, terrace, deck or patio, and from a height of 1.7 metres above floor level. A habitable room window, balcony, terrace, deck or patio that is located with a direct view into a habitable room window of existing dwelling or small second dwelling within a horizontal distance of 9 metres (measured at ground level) of the window, balcony, terrace, deck or patio: - Is offset a minimum of 1.5 metres from the edge of one window to the edge of the other; or - Has sill heights of at least 1.7 metres above floor level; or - Has fixed, obscure glazing in any part of the window below 1.7 metre above floor level; or - Has permanently fixed external screens to at least 1.7 metres above floor level and be no more than 25 per cent transparent; or - Has fixed elements that prevent direct views, such as horizontal ledges or vertical fins. Obscure glazing in any part of the window below 1.7 metres above floor level may be openable provided that there are no direct views as specified in this standard. Screens used to obscure a view should be: - Perforated panels or trellis with a maximum of 25 per cent openings or solid translucent panels. - Permanent, fixed and durable. - Designed and coloured to blend in with the development. This standard does not apply to a new habitable room window, balcony, terrace, deck or patio which faces a property boundary where there is a visual barrier at least 1.8 metres high and the floor level of the habitable room, balcony, terrace, deck or patio is less than 0.8 metres above ground level at the boundary. | Deemed to Comply The development is designed to meet the requirements of the standard with overlooking into existing secluded private open space areas and habitable room windows limited in accordance with the Standard in the form of: INSERT Not Applicable There are no habitable room windows or secluded private open space on adjoining land that require consideration. Variation INSERT Before deciding on an application, the responsible authority must consider: - The design response. - The impact on the amenity of the secluded private open space or habitable room window. - The existing extent of overlooking into the secluded private open space and habitable room windows of existing dwellings. - The internal daylight to and amenity of the proposed dwelling or residential building. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met Yes, standard varied |
| 54.05 Sustainability |  |  |  |
| 54.05-1 Permeability objectives To reduce the impact of increased stormwater run-off on the drainage system. To facilitate on-site stormwater infiltration. To contribute to urban cooling | Standard A5-1 The site area covered by the pervious surfaces is at least 20 percent of the site. | Deemed to Comply The site retains INSERT per cent permeability, assisting in on site stormwater filtration. Variation INSERT Before deciding on an application, the responsible authority must consider: - The design response. - The existing site coverage and any constraints imposed by existing development. - The capacity of the drainage network to accommodate additional stormwater. - The practicality of achieving the minimum site coverage of pervious surfaces. INSERT Does not meet the objective INSERT | Right of Appeal No |
| 54.05-2 Overshadowing domestic solar energy systems To ensure that the height and setback of a building from a boundary allows reasonable solar access to existing domestic solar energy systems on the roofs of buildings | Standard A5-2 Any part of a new building that will reduce the sunlight at any time between 9am and 4 pm on 22 September to an existing domestic solar energy system on the roof of a building on an adjoining lot be set back from the boundary to that lot by at least 1 metre at 3.6 metres above ground level, plus 0.3 metres for every metre of building height over 3.6 metres up to 6.9 metres, plus 1 metre for every metre of height over 6.9 metres. This standard applies to an existing building in a Township Zone, General Residential Zone or Neighbourhood Residential Zone. In Clause 54.05-2, domestic solar energy system means a domestic solar energy system that existed at the date the application was lodged. | N/A There are no existing domestic solar energy systems on adjoining buildings that require consideration. or The site is located in the Residential/Mixed Use Zone/Housing Choice and Transport Zone. Deemed to Comply There is an existing domestic solar system on the adjoining building to the INSERT. Overshadowing diagrams demonstrate that the domestic solar system will not be overshadowed at anytime between 9am and 4pm on 22 September. or Overshadowing diagrams demonstrate that the domestic solar system will be overshadowed between 9am and 4pm on 22 September. The building has been setback INSERT metres in accordance with the Standard. Variation INSERT Decision guidelines Before deciding on an application, the responsible authority must consider whether the domestic solar energy system has been sited to optimise efficiency and protection from overshadowing. INSERT Does not meet the objective INSERT | Right of Appeal No, standard met Yes, standard varied |
| 54.05-3 Rooftop solar energy generation area objective To support the future installation of appropriately sited rooftop solar energy systems for a dwelling. | Standard A5-3 In Clause 54.05-3, rooftop solar energy area means an area provided on the roof of a dwelling to enable the future installation of a solar energy system. An area on the roof is capable of siting a rooftop solar energy area for each dwelling which: - Has a minimum dimension of 1.7 metres. - Has a minimum area in accordance with Table B5-3. - Is oriented to the north, west or east. - Is positioned on the top two thirds of a pitched roof. - Can be a contiguous area or multiple smaller areas. - Is free of obstructions on the roof of the dwelling within twice the height of each obstruction (H), measured horizontally (D) from the centre point of the base of the obstruction to the nearest point of the rooftop solar energy area. | N/A This Standard does not apply to a small second dwelling. Deemed to Comply The submitted plans demonstrate that each dwelling: - Has a minimum dimension of 1.7 metres. - Has a minimum area of INSERT sqm in accordance with Table B5-3. - Is oriented to the north, west or east. - Is positioned on the top two thirds of a pitched roof. - Is an contiguous area or multiple smaller areas. - Is free of obstructions on the roof of the dwelling within twice the height of each obstruction (H), measured horizontally (D) from the centre point of the base of the obstruction to the nearest point of the rooftop solar energy area. Variation INSERT Decision guidelines Before deciding on an application, the responsible authority must consider: - The design response. - The size and orientation of the building. - The availability of solar access to the rooftop. - The extent to which the rooftop solar energy generation area is overshadowed by existing buildings, other permanent structures or equipment on the rooftop. INSERT Does not meet the objective INSERT | Right of Appeal No |
| 54.05-4 Solar protection to new north facing windows objective To encourage external shading of north facing windows to minimise summer heat gain | Standard A5-4 North facing windows are shaded by eaves, fixed horizontal shading devices or fixed awnings with a minimum horizontal depth of 0.25 times the window height. | N/A This Standard does not apply to a small second dwelling. Deemed to Comply North facing windows are shaded by eaves, fixed horizontal shading devices or fixed awnings with a minimum horizontal depth of INSERT metres being 0.25 x the window height of INSERT metres. Variation INSERT Before deciding on an application, the responsible authority must consider: - The design response. - The size and orientation of the lot. - The type and useability of external solar shading devices, including alternative design responses. INSERT Does not meet the objective INSERT | Right of Appeal No |

…………………………

INSERT – Planning Officer

………………………….

Team Leader